ISO

International Organization for Standardization

Organisation Internationale de Normalisation

ISO/IEC JTC1/SC2/WG2

Universal Coded Character Set (UCS)

ISO/IEC JTC1/SC2/WG2 N

1 October, 2026

Title:		Defect report on ISO/IEC 10646-1:1993 for representing the C1 control characters

Source:	USA (ANSI)

Distribution:	ISO/IEC JTC 1/SC 2/WG 2

Requested Action:	Correction of Technical Defect in ISO/IEC 10646-1:1993

10.  Nature of defect

ISO/IEC 10646-1:1993 requires that the C1 control characters be coded only as ESC Fe sequences.

Rationale

This is unduly restrictive.  It fails to exploit the flexibility for representing the C1 control characters allowed in ISO/IEC 6429.  It makes conversion of 8-bit coded-characters-sets compliant with ISO/IEC 4873 into the ISO/IEC 10646-1 code unnecessarily complicated.

In accordance with ISO/IEC 6429, the C1 control characters may be represented as either the C1 control-character code positions or ESC Fe sequences.  By requiring that the C1 control characters be encoded exclusively as ESC Fe sequences, 10646 is unnecessarily restrictive.

This ISO/IEC 10646-1 restriction makes conversion between 8-bit coded-character-sets compliant with ISO/IEC 4873 and ISO/IEC 10646-1 unnecessarily complicated.  ISO/IEC 8859-1 is an 8-bit coded-character-set that complies with ISO/IEC 4873.  Translating the graphic characters of ISO 8859-1 into the 10646-1 code is a direct character-for-character translation.  The same is true for 8-bit control characters from the C0 area of ISO/IEC 8859-1 and any C1 control characters encoded as ESC Fe sequences.  However converting from each 8-bit C1 control character directly encoded using the C1 code positions into ISO/IEC 10646-1 requires that the CC data element be expanded by one character position to accommodate the two-character ESC Fe encoding of the C1 control characters in ISO/IEC 10646.

In addition, if ISO/IEC 10646 were to allow the C1 control characters to be directly encoded using the C1 code positions, then software could process the C1 control characters the same way as the C0 control characters, that is, as single characters instead of two-character ESC Fe sequences.  Allowing ISO/IEC 10646-1 to represent the C1 control characters as either the C1 code positions or ESC Fe sequences would enable direct one-to-one code-conversion between data coded in 8-bit coded-character-sets compliant with ISO/IEC 4873 and data coded in ISO/IEC 10646-1.

Finally, it is our understanding that the representation of the C1 control characters was specified as ESC Fe sequences so that the C1 control character area in ISO/IEC 10646-1 would be available for the encoding of certain (as yet unspecified) graphic characters found in some PC code pages.  However, all important characters in PC code pages have already been coded in ISO/IEC 10646-1 outside of the C1 space.   That is, the C1 space in ISO/IEC 10646-1 need not be reserved for graphic characters, and thus can be released for the direct encoding of C1 control characters of ISO/IEC 6429.

11.  Solution proposed by the submitter

The requirement that C1 control characters be encoded as only ESC Fe sequences should be removed.  In accordance with ISO/IEC 6429, the C1 control characters should be allowed to be encoded both as the C1 code positions reserved for them in ISO/IEC 10646-1 and the ESC Fe sequences as currently specified.

This may be effected with the following changes of ISO/IEC 10646-1:1993:

Clause 8

(a)	at the end of paragraph 3, append "for control characters" so that the sentence reads:

Code positions 0080 to 009F are reserved for control characters.

Clause 16

(a)	remove the term "C0" from paragraph 2.

(b)	delete paragraphs 6 and 7.